# Brief Report: Channel Estimation with LS, LMMSE and CNN

# 1. Objective

Implement and compare a basic channel estimation process (LS and LMMSE) with a simple CNN-based estimator, using NMSE and BER as the main metrics.

# 2. System Setup

- Link: OFDM with 64 subcarriers and an all-pilot block (QPSK pilots).

- Channel: Rayleigh multipath with 8 taps and an exponential power-delay profile (decay 0.5), assumed shorter than the cyclic prefix.

- Noise: AWGN, evaluated at SNR 0 to 30 dB in 5 dB steps.

# 3. Methods

- LS: H = Y/X. It needs no prior knowledge and does no denoising.

- LMMSE: H = R(R + σ²I)⁻¹ H_LS. It uses the exact channel covariance and the SNR.

- CNN:

- A 5-layer 1D CNN with 64 filters, kernel size 5 and circular padding.

- Input is Re/Im of the LS estimate plus an SNR channel; it predicts a residual correction on LS.

- Trained for 30 epochs with Adam and MSE loss on freshly generated data at random SNRs from 0 to 30 dB.

# 4. Metrics

- NMSE: ‖H − Ĥ‖² / ‖H‖², in dB.

- BER: QPSK data equalized with ZF using each estimate, with perfect CSI as a reference.

- Also reported: parameter count and inference time.

# 5. Results

| SNR | NMSE LS | NMSE LMMSE | NMSE CNN |
| --- | --- | --- | --- |
| 0 dB | ≈ 0 dB | ≈ −10 dB | ≈ −9.66 dB |
| 10 dB | ≈ −10 dB | ≈ −19 dB | ≈ −19.5 dB |
| 30 dB | ≈ −30 dB | ≈ −39 dB | ≈ −35.62 dB |

NMSE values for the CNN are read from the plot.

- LS NMSE equals −SNR, as expected.

- LMMSE gains about 9 dB over LS, matching the L/N = 8/64 theory.

- The CNN comes close to LMMSE at low and mid SNR, and the gap grows to about 3 to 4 dB at high SNR.

- In BER, LMMSE, CNN and perfect CSI nearly overlap and all beat LS.

# 6. Observations

- The CNN reaches near-LMMSE performance without being given the covariance. It learned the channel structure from data.

- The CNN lags at high SNR because plain MSE loss is dominated by low-SNR training samples.

- The training loss is above the validation loss because training spans 0 to 30 dB while validation is at 10 dB only. This is not overfitting.

- LMMSE has an advantage here because it is given the exact covariance, which is rarely known in practice.

# 7. Limitations and Next Steps

- The estimators have been tested only on the same channel statistics they were designed or trained for.

- Robustness test: train at DECAY = 0.5 and test with different delay profiles or tap counts. LMMSE keeps its fixed covariance, so this shows where it breaks and how the CNN generalizes.

- Loss function: use a normalized per-sample loss to improve high-SNR accuracy.

- Pilot pattern: try comb-type pilots with interpolation as the baseline.

Code: MLProject.ipynb attached.