# Modulation Recognition: Handcrafted Features vs CNN

Chapter 3 assignment, option 2: classify BPSK, QPSK, 16-QAM and 64-QAM

# 1. Task and Scenario

A receiver gets a burst of symbols with an unknown modulation and must classify it as BPSK, QPSK, 16-QAM or 64-QAM. Symbol timing is assumed recovered (one sample per symbol), and each sample is a window of 128 symbols. The objective is high accuracy across SNR and channel conditions, including conditions not seen in training.

# 2. Signal Model and Data

- Model: r[n] = h·exp(j(2πfn + φ))·s[n] + w[n], with unit-power symbols s[n], a flat block-fading gain h (constant within a burst), unknown phase φ ~ U(0, 2π), carrier-frequency offset (CFO) f, and noise w ~ CN(0, 10^(−SNR/10)). Each window is normalized to unit power (AGC).

- Channels: AWGN (h = 1), Rayleigh (h ~ CN(0,1)) and Rician (K = 6 dB), with E|h|² = 1 so that SNR is the average SNR.

- Training data: AWGN and Rayleigh, SNR −5 to 20 dB in 5 dB steps, no CFO. 150 bursts per modulation, SNR and channel; each burst gives 4 windows (28,800 windows in total).

- Unseen test conditions: Rician fading, SNR from −10 to 30 dB (below and above the training range), and CFO up to 0.005 cycles/symbol at 15 dB. These test sets are generated fresh with a separate random stream.

# 3. Features, Baseline and ML Method

- Baseline: seven handcrafted features (|C20|, |C40|, |C41|, C42, E|r|⁴, E|r|⁶, std of |r|), which ignore the unknown phase, classified by a 200-tree random forest.

- ML method: a small CNN (38,980 parameters, three conv blocks) on a 32×32 constellation image, a 2-D histogram of the received I/Q points. The CNN suits the task because the information is the shape of the point cloud. Training used Adam for 15 epochs, with the epoch chosen on validation bursts.

- Both methods use the same windows, splits and test sets, so only the classifier differs.

# 4. Metrics and Leakage Prevention

- Metrics: accuracy vs SNR, confusion matrices at 0, 10 and 20 dB, and accuracy vs CFO. All experiments use 3 seeds (0, 1, 2) and are reported as mean ± std.

- Leakage prevention: the 4 windows of a burst share the same channel gain, phase and SNR, so the pool is split by whole burst (60/20/20), and the code asserts no burst appears in two splits. The CNN epoch is selected on validation bursts only, and no preprocessing is fitted on test data.

# 5. Results

| Condition (accuracy %) | Baseline: cumulants + RF | Proposed: CNN |
| --- | --- | --- |
| Held-out bursts (training conditions) | 65.3 ± 0.7 | 68.1 ± 0.7 |
| AWGN, 0 dB | 48.1 ± 1.8 | 48.5 ± 1.1 |
| AWGN, 10 dB | 72.5 ± 1.1 | 74.8 ± 0.2 |
| AWGN, 20 dB | 89.8 ± 1.0 | 97.5 ± 0.8 |
| Rayleigh, 20 dB | 84.3 ± 1.7 | 93.0 ± 2.0 |
| Rician (unseen), 20 dB | 87.6 ± 1.1 | 96.4 ± 1.9 |
| AWGN, 30 dB (above training range) | 86.9 ± 2.4 | 97.5 ± 1.9 |
| AWGN, −10 dB (below training range) | 28.3 ± 1.0 | 26.8 ± 1.0 |
| CFO max 0.001, AWGN 15 dB | 84.4 ± 0.5 | 93.9 ± 0.1 |
| CFO max 0.002, AWGN 15 dB | 81.7 ± 0.4 | 86.2 ± 0.7 |
| CFO max 0.005, AWGN 15 dB | 67.2 ± 0.8 | 67.1 ± 0.7 |

Mean ± std over 3 seeds. Chance level is 25%.

Accuracy vs SNR (shaded: training range). Baseline in orange, CNN in blue.

- The CNN is clearly better at medium to high SNR (about +8 to +9 points at 20 dB on AWGN, Rayleigh and unseen Rician), and about equal to the baseline at 0 dB and below.

- Fading lowers accuracy (Rayleigh 20 dB: 93.0% vs 97.5% on AWGN) because a deep fade lowers the effective SNR of a whole burst. The unseen Rician channel behaves like AWGN, so the CNN generalized across these fading types.

- Leakage check (seed 0): a random window split raised accuracy from 62.7% to 63.9% for the baseline and from 64.8% to 67.5% for the CNN, although 4,114 bursts were shared between train and test. The effect is modest, but it shows that a random split overstates performance.

- Cost: the CNN needs 0.19 ms per window vs 0.04 ms for the baseline (including feature or image creation).

# 6. Failure Analysis

- Low SNR, constellation overlap: at 0 dB, noise blurs the points of QPSK, 16-QAM and 64-QAM into one cloud. BPSK is still recognized perfectly, but the CNN labels 82% to 86% of the other three classes as 64-QAM, and the baseline spreads its errors among them. At 10 dB, 16-QAM and 64-QAM are still confused (the CNN labels 95% of 16-QAM as 64-QAM), and at 20 dB the CNN separates all classes except some 64-QAM errors.

- CFO: with no CFO in training, accuracy drops as the offset grows (CNN 97.9% at 0, 86.2% at 0.002, 67.1% at 0.005) because the rotating constellation smears into rings. Both methods fail the same way at large CFO, which points to the signal physics, not the classifier.

- SNR outside the training range: at −10 dB both methods are near chance (about 27%). At 30 dB the CNN stays at 97.5%, but the baseline stays at 86.9%, most likely because 128 symbols give noisy cumulant estimates for 16-QAM and 64-QAM.

# 7. Limitations

- All data is simulated: flat block fading, no frequency selectivity, perfect symbol timing, no phase noise or hardware impairments, and no CFO in training.

- Closed set of four classes, with no rejection of unknown signals; one feature set and one CNN size were tried.

- Next steps: train with CFO and run a CFO estimator first, add an unknown-signal reject class (open-set AMC), and test on measured signals.

# 8. Code Appendix

Code: modulation_recognition_assignment.ipynb (Google Colab, runs with Runtime > Run all; about 6 minutes on CPU). All parameters are in the first code cell: seeds 0, 1, 2; 128 symbols per window; training SNR −5 to 20 dB; 150 bursts per cell; 15 epochs; batch 128; learning rate 1e-3. Software: Python 3.12, NumPy 2.4, scikit-learn 1.8, PyTorch 2.14. The notebook writes results_main_table.csv and the figures fig_accuracy_vs_snr.png, fig_confusion.png, fig_cfo.png and fig_leakage_training.png.